Tugas Pemulihan Jaringan Error:

- Lakukan ping test antar perangkat (PC1 ke PC2 dan ke Server).

- Catat hasil error yang muncul dan identifikasi kemungkinan penyebab.

- Perbaiki semua gangguan agar jaringan kembali normal.

- Buat laporan singkat berisi:

  - Jenis gangguan

  - Penyebab gangguan

  - Langkah perbaikan

  - Tindakan pencegahan

Tabel Configuration Network

NOTE: HOST NEED IS BASED ON UNIT + GATEWAY FOR ACCROS TRAFIC

NOTE: SUBNETMASK FROM /CIDR = /30 = 4IP = 256 BITS – 4 = 252

| Subnet | Kebutuhan Host | Prefix | Network ID | Range Host | Broadcast | SUBNET MASK |
| --- | --- | --- | --- | --- | --- | --- |
| A | 1 | /30 | 192.168.10.0 | 192.168.10.1 - 192.168.10.2 | 192.168.10.3 | 255.255.255.252 |
| B | 2 | /30 | 192.168.10.4 | 192.168.10.5 - 192.168.10.6 | 192.168.10.7 | 255.255.255.252 |

| RUANG | UNIT | IP ADDRESS | SUBNET MASK | DEFAULT GATEWAY |
| --- | --- | --- | --- | --- |
| RUANG A | PC-1 | 192.168.10.2 | 255.255.255.252 | 192.168.10.1 |
| RUANG B | PC-2 | 192.168.10.6 | 255.255.255.252 | 192.168.10.5 |
|  | SERVER | 192.168.10.5 | 255.255.255.252 | 192.168.10.5 |

ILUSTRATIONS:

NETWORK RESPONSE:

RESPONSE ROUTER INTO DEVICE CONNECTION

RESPONSE DEVICE INTO DEVICE CONNECTION

Tabel NETWORK LOGGING

| ERROR-ID | ROOT CAUSE | RELATION | LOG-ERR | TYPE ISSUE | STATUS(WIP/DONE) | ACTION | PREVENTION ACTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ERR-1 | PC-2 is missing the IP Address and Subnet Mask configuration. | PC-1-TO-PC-2 ROUTER-PC-2 | PC2 HAS NO FUNCTIONAL PORTS. | Configuration Error | DONE | - Set up the CIDR as required. - Assign the IP and Subnet Mask to the PC2 port based on the established configuration. - Perform a Ping Test to confirm cross-device connectivity. | Analyze the required number of IP Address and setup the network configuration use VLSM Implement Integration test, by testing connection between cross device |
| ERR-2 | SERVER HAS MISSMATCH IP,SUBNET-MASK AND EMPTY GATEWAY | PC-1-TO-SERVER ROUTER-SERVER | FAILED | CONFIGURATION ERROR | DONE | - Set up the CIDR as required. - Assign the IP and Subnet Mask to the PC2 port based on the established configuration. - Perform a Ping Test to confirm cross-device connectivity. | Analyze the required number of IP Address and setup the network configuration use VLSM Implement Integration test, by testing connection between cross device |
| ERR-3 | ROUTER HAS LENGTH IP ADDRES AND SUBNET MASK NOT IDEAL | ROUTER |  | PEFORMANCE | DONE | - SETUP IP ADDRES AND SUBNET MASK BASED ON VLSM Best Pratice - Assign the IP and Subnet Mask to the PC2 port based on the established vlsm configuration. - Perform a Ping Test to confirm cross-device connectivity. | Analyze the required number of IP Address and setup the network configuration use VLSM |